CN Project

Part 1:

Reorder:

| Subnet description | VLAN ID | VLAN Name | Required Number of Hosts | Network Address/CIDR | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VLAN 35 (S) |  | 61 | 10.0.0.0 /26 | 10.0.0.1 /26 | 10.0.0.62 /26 | 10.0.0.63 /26 |
|  | VLAN 15 (Main) |  | 31 | 10.0.0.64 /26 | 10.0.0.65 /26 | 10.0.0.126 /26 | 10.0.0.127 /26 |
|  | VLAN 45 (N) |  | 30 | 10.0.0.128 /27 | 10.0.0.129 /27 | 10.0.0.158 /27 | 10.0.0.159 /27 |
|  | VLAN 5 (Main) |  | 25 | 10.0.0.160 /27 | 10.0.0.161 /27 | 10.0.0.190 /27 | 10.0.0.191 /27 |
|  | VLAN 65 (R) |  | 20 | 10.0.0.192 /27 | 10.0.0.193 /27 | 10.0.0.222 /27 | 10.0.0.223 /27 |
|  | VLAN 130 (S) |  | 12 | 10.0.0.224 /28 | 10.0.0.225 /28 | 10.0.0.238 /28 | 10.0.0.239 /28 |
|  | VLAN 55 (N) |  | 12 | 10.0.0.240 /28 | 10.0.0.241 /28 | 10.0.0.254 /28 | 10.0.0.255 /28 |
|  | VLAN 75 (R) |  | 7 | 10.0.1.0 /28 | 10.0.1.1 /28 | 10.0.1.14 /28 | 10.0.1.15 /28 |
|  | SW-S to MIU-MIU-GW |  | 7 | 10.0.1.16 /28 | 10.0.1.17 /28 | 10.0.1.30 /28 | 10.0.1.31 /28 |
|  | Main-MLS to MIU-MIU-GW |  | 2 | 10.0.1.32 /30 | 10.0.1.33 /30 | 10.0.1.34 /30 | 10.0.1.35 /30 |
|  | N-MLS to MIU-MIU-GW |  | 2 | 10.0.1.36 /30 | 10.0.1.37 /30 | 10.0.1.38 /30 | 10.0.1.39 /30 |
|  | S-MLS to MIU-MIU-GW |  | 2 | 10.0.1.40 /30 | 10.0.1.41 /30 | 10.0.1.42 /30 | 10.0.1.43 /30 |
|  | R-MLS to MIU-MIU-GW |  | 2 | 10.0.1.44 /30 | 10.0.1.45 /30 | 10.0.1.46 /30 | 10.0.1.47 /30 |

# Subnet Table

| Subnet Description | VLAN ID | VLAN Name | Required Number of Hosts | Network Address/CIDR | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Main | VLAN 5 |  | 25 | 10.0.0.160 /27 | 10.0.0.161 /27 | 10.0.0.190 /27 | 10.0.0.191 /27 |
|  | VLAN 15 |  | 31 | 10.0.0.64 /26 | 10.0.0.65 /26 | 10.0.0.126 /26 | 10.0.0.127 /26 |
| S | VLAN 130 |  | 12 | 10.0.0.224 /28 | 10.0.0.225 /28 | 10.0.0.238 /28 | 10.0.0.239 /28 |
|  | VLAN 35 |  | 61 | 10.0.0.0 /26 | 10.0.0.1 /26 | 10.0.0.62 /26 | 10.0.0.63 /26 |
| N | VLAN 45 |  | 30 | 10.0.0.128 /27 | 10.0.0.129 /27 | 10.0.0.158 /27 | 10.0.0.159 /27 |
|  | VLAN 55 |  | 12 | 10.0.0.240 /28 | 10.0.0.241 /28 | 10.0.0.254 /28 | 10.0.0.255 /28 |
| R | VLAN 65 |  | 20 | 10.0.0.192 /27 | 10.0.0.193 /27 | 10.0.0.222 /27 | 10.0.0.223 /27 |
|  | VLAN 75 |  | 7 | 10.0.1.0 /28 | 10.0.1.1 /28 | 10.0.1.14 /28 | 10.0.1.15 /28 |
| From Main-MLS to MIU-MIU-GW | - |  | 2 | 10.0.1.32 /30 | 10.0.1.33 /30 | 10.0.1.34 /30 | 10.0.1.35 /30 |
| From N-MLS to MIU-MIU-GW | - |  | 2 | 10.0.1.36 /30 | 10.0.1.37 /30 | 10.0.1.38 /30 | 10.0.1.39 /30 |
| From S-MLS to MIU-MIU-GW | - |  | 2 | 10.0.1.40 /30 | 10.0.1.41 /30 | 10.0.1.42 /30 | 10.0.1.43 /30 |
| From R-MLS to MIU-MIU-GW | - |  | 2 | 10.0.1.44 /30 | 10.0.1.45 /30 | 10.0.1.46 /30 | 10.0.1.47 /30 |
| From SW-S to MIU-MIU-GW | - |  | 7 | 10.0.1.16 /28 | 10.0.1.17 /28 | 10.0.1.30 /28 | 10.0.1.31 /28 |

# Addressing Table

| Device | Interface | Address | Subnet Mask | Default Gateway | VLAN ID |
| --- | --- | --- | --- | --- | --- |
| Main-MLS | G1/1/1 | 10.0.1.33 /30 | 255.255.255.252 | — | — |
|  | VLAN 5 | 10.0.0.161 /27 | 255.255.255.224 | — | 5 |
|  | VLAN 15 | 10.0.0.65 /26 | 255.255.255.192 | — | 15 |
| S-MLS | G1/1/1 | 10.0.1.41 /30 | 255.255.255.252 | — | — |
|  | VLAN 130 | 10.0.0.225 /28 | 255.255.255.240 | — | 130 |
|  | VLAN 35 | 10.0.0.1 /26 | 255.255.255.192 | — | 35 |
| N-MLS | G1/1/1 | 10.0.1.37 /30 | 255.255.255.252 | — | — |
|  | VLAN 45 | 10.0.0.129 /27 | 255.255.255.224 | — | 45 |
|  | VLAN 55 | 10.0.0.241 /28 | 255.255.255.240 | — | 55 |
| R-MLS | G1/0/5 | 10.0.1.45 /30 | 255.255.255.252 | — | — |
|  | VLAN 65 | 10.0.0.193 /27 | 255.255.255.224 | — | 65 |
|  | VLAN 75 | 10.0.1.1 /28 | 255.255.255.240 | — | 75 |
| MIU-MIU-GW | Gig0/0 | 209.165.200.226 /28 | 255.255.255.240 | — |  |
|  | Gig0/1 | 10.0.1.46 /30 | 255.255.255.252 | — |  |
|  | Gig0/0/0 | 10.0.1.17 /28 | 255.255.255.240 | — |  |
|  | Gig0/1/0 | 10.0.1.34 /30 | 255.255.255.252 | — |  |
|  | Gig0/2/0 | 10.0.1.42 /30 | 255.255.255.252 | — |  |
|  | Gig0/3/0 | 10.0.1.38 /30 | 255.255.255.252 | — |  |
| ISP | Gig 0/0 | 209.165.200.225 | 255.255.255.240 | — |  |
|  | Gig 0/1 | 64.100.1.1 | 255.255.255.224 | — |  |
|  | Gig 0/2 | 64.100.2.1 | 255.255.255.224 | — |  |
| Branch-GW | Gig 0/0.2 | 192.168.2.1 | 255.255.255.0 | — |  |
|  | Gig 0/0.3 | 192.168.3.1 | 255.255.255.0 | — |  |
|  | Gig 0/1 | 64.100.1.2 | 255.255.255.224 | — |  |
| Wireless Home Router | G0/1 | 64.100.2.2 | 255.255.255.224 | — |  |
|  | Router IP | 192.168.10.1 | 255.255.255.128 | — |  |

# Hosts Address

| PC-number | VLAN ID | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| MIU |  |  |  |  |
| PC-1 | VLAN 5 | From DHCP Server 10.0.0.162 /27 | From DHCP Server 255.255.255.224 | From DHCP Server 10.0.0.161 |
| PC-2 | VLAN 5 | From DHCP Server 10.0.0.163 /27 | From DHCP Server 255.255.255.224 | From DHCP Server 10.0.0.161 |
| PC-3 | VLAN 15 | From DHCP Server 10.0.0.67 /26 | From DHCP Server 255.255.255.192 | From DHCP Server 10.0.0.65 |
| PC-4 | VLAN 130 | From DHCP Server 10.0.0.226 /28 | From DHCP Server 255.255.255.240 | From DHCP Server 10.0.0.225 |
| PC-5 | VLAN 130 | From DHCP Server 10.0.0.227 /28 | From DHCP Server 255.255.255.240 | From DHCP Server 10.0.0.225 |
| PC-6 | VLAN 35 | From DHCP Server 10.0.0.2 /26 | From DHCP Server 255.255.255.192 | From DHCP Server 10.0.0.1 |
| PC-7 | VLAN 45 | 10.0.0.130 /27 | 255.255.255.224 | 10.0.0.129 |
| PC-8 | VLAN 55 | 10.0.0.242 /28 | 255.255.255.240 | 10.0.0.241 |
| PC-9 | VLAN 65 | 10.0.0.194 /27 | 255.255.255.224 | 10.0.0.193 |
| PC-10 | VLAN 75 | 10.0.1.2 / 28 | 255.255.255.240 | 10.0.1.1 |
| MIU_ Branch 1 |  |  |  |  |
| PC-11 |  | From DHCP Server 192.168.2.3 | From DHCP Server 255.255.255.0 | From DHCP Server 192.168.2.1 |
| PC-12 |  | From DHCP Server 192.168.3.3 | From DHCP Server 255.255.255.0 | From DHCP Server 192.168.3.1 |
| Wireless Home Network |  |  |  |  |
| Laptop |  | 192.168.10.10/25 | 255.255.255.128 | 192.168.10.1 |
| Tablet |  | 192.168.10.30/25 | 255.255.255.128 | 192.168.10.1 |
| Smartphone |  | 192.168.10.20/25 | 255.255.255.128 | 192.168.10.1 |
| Servers |  |  |  |  |
| DHCP Server |  | 10.0.1.18 /28 | 255.255.255.240 | 10.0.1.17 |
| Email Server |  | 10.0.1.19 /28 | 255.255.255.240 | 10.0.1.17 |
| Web server |  | 10.0.1.20 /28 | 255.255.255.240 | 10.0.1.17 |
| DNS server |  | 10.0.1.21 /28 | 255.255.255.240 | 10.0.1.17 |
| NTP and Syslog server |  | 10.0.1.22 /28 | 255.255.255.240 | 10.0.1.17 |

Part 2: Build the Network and Configure Basic Device Settings and Interface Addressing. (10%)

In Part 2, you will set up the network topology and configure basic settings and interface addressing.

Step 1: Configure PCs with IPv4 addresses

Use the addressing table to manually configure the PCs with full IP addressing.

manually as per above.

Step 2: Configure basic settings for each device.

a.     Configure all Devices with the following:

R(Config)# no ip domain-lookup

2)     Host name for all devices.

Router(config)#hostname router_name

3)     Set the minimum password length to 10 characters.

R(Config)# Security passwords min-length 10

4)     Use the MIU1234567 password for Console access password.

Router(config)#line con 0

Router(config-line)#login

Router(config-line)#password MIU1234567

5)     Use the CSC1234567 secret password.

Router(config)#enable secret CSC1234567

6)     Encrypt the clear text passwords.

R(Config)# service password-encryption

7)     Configure an appropriate MOTD Banner.

Router(config)#banner motd “type message here”

b.     Configure addressing for all devices according to the Addressing Table.

  Configuring IP addresses on the network devices (Routers, Switches, Layer 3 Switches).

  Assigning IP addresses to:

- Router interfaces (e.g., G0/0, G1/1)

- SVIs (Switched Virtual Interfaces on Layer 3 switches)

- L3 interfaces if using a multi-layer switch for inter-VLAN routing

- Configuring a Gigabit Ethernet Interface with IPv4:

Router(config)#interface gigabitethernet 0/0

Router(config-if)#description Accounting LAN

Router(config-if)#ip address 192.168.20.1 255.255.255.0

Router(config-if)#no shutdown

- Inter-VLAN Communication Using an External Router: Router-on-a-Stick:

Router(config)#interface gigabitethernet 0/0

Router(config-if)#no shutdown

Router(config-if)#exit

c.      Configure SSH for all routers.

R(config) # ip domain-name cisco.com

R(config) # crypto key generate rsa press Enter 1024

R(config) # username Bop privilege 15 Secret Class or(R(config) # username Bop password Class)

R(config) # Line vty 0 4

R(config-line) # login local

R(config-line) # transport input ssh

R(config-line) # exit

Part 3:

Configure Network Infrastructure Settings (VLANs, Trunking, Inter-VLAN Routing

EtherChannel). (10%)

In this part, you will complete the Layer 2 network configuration and set up basic host support. At the end of this part, all the

switches should be able to communicate.

Step 1: Create VLANs on all switches.

S1(config)# vtp mode transparent

S1(config)# VLAN 2000

S1(config-vlan)# end

Step 2: On all switches, configure host access ports connecting to PCs.

  - Issue the show vlan brief command to verify VLAN configuration.

Switch(config)#interface range fastethernet 0/1 – 9

Switch(config-if-range)#switchport mode access

Switch(config-if-range)#switchport access vlan 10

Step 3: Configure an 802.1Q Trunk between the Switches

Switch(config)#interface fastethernet 0/1

Switch(config-if)#switchport mode trunk

Switch(config-if)#switchport trunk native VLAN 99

Switch(config-if)#switchport trunk allowed VLAN 10,20,30,99

Step 4: Configure Inter-VLAN Routing:

a.       on the L3 Switches in the Main Branch:

§ On all L3 Switches, configure the interface connected to MIU-MIU-GW as a routed port and assign an IP

address according to the Addressing Table

§ Configure and activate the SVI interfaces for VLANs according to the Addressing Table

b.       on the router in the MIU Branch:

§ Configure sub interfaces on Branch-GW router using the 802.1Q encapsulation.

- Set the encapsulation type to 802.1Q and assign VLAN 10 to the sub interface.

- Refer to the Address Table and assign the correct IP address to the sub interface.

Step 5: Verify Inter-VLAN Routing is working

Use ping trace route and arp table

Step 6: On all switches, create LACP EtherChannels as shown in the topology diagram.

S1(config)#interface range fastethernet 0/1 – 2

S1 (config-if)#channel-group 1 mode Active

S1 (config-if)#exit

• S1(config)#interface port-channel 1

S1(config-if)# Switchport mode Trunk

S1(config-if)# switchport trunk native vlan 999

S1(config-if)# switchport trunk allowed vlan 1,10,20

Step 7: Verify EtherChannels are working.

show vlan brief

show interfaces trunk

show etherchannel summary

show ip interface brief

ping [other device IP]

Part 4:

Configure Routing Protocols. (10%)

In this part, you will configure IPv4 routing protocols. At the end of this part, the network should be fully converged.

Step 1: Point-to-Point Single-Area OSPFv2 Configuration

- Start the OSPF routing process on MIU-GW router, Main-MLS, and S-MLS. Use process ID 100.

R2(config)#router ospf 100

- Configure Router IDs for three routers.

Router(config-router)# router-id rid

MIU-GW = 1.1.1.1

Main-MLS = 2.2.2.2

S-MLS = 3.3.3.3

- Configure Networks for OSPF Routing for three routers.

R2(config-router)# network interfaceIP 0.0.0.0 area 0

d.      Configure Passive Interfaces to prevent the OSPF process from sending unnecessary routing protocol traffic out

LAN interfaces.

Router(configrouter)# passive-interface port

- Configure OSPF to propagate the default route in OSPF routing updates on MIU-GW router.

Router(config)#ip route 0.0.0.0 0.0.0.0 port

Router(config)#router ospf 100

Router(config-router)#defaultinformation originate

- Verify OSPF configuration. Use show commands to verify the network and passive interface configuration of the OSPF process on each router.

show ip ospf neighbor

• show ip ospf

• show ip ospf interface

• Show ip protocols

• show ip ospf interface brief

• show ip route ospf

• show ip ospf database

Step 2: Basic EIGRP with IPv4 Configuration

- Enable the EIGRP routing process on MIU-GW router, N-MLS, and R-MLS, using AS number 10.

conf t

router eigrp 10

no auto-summary

- Advertise directly connected networks on three routers.

router eigrp 10

network 192.168.1.0 0.0.0.255

network 10.1.1.0 0.0.0.255

- Configure the LAN interfaces to not advertise EIGRP updates.

router eigrp 10

passive-interface GigabitEthernet0/1

- Configure EIGRP to propagate the default route in OSPF routing updates on MIU-GW router.

ip route 0.0.0.0 0.0.0.0 <next-hop-ip>

router eigrp 10

redistribute static

- Verify EIGRP Routing

show ip eigrp neighbors

show ip route eigrp

show ip route

show ip protocols

Step 3: Configure Route Redistribution Between EIGRP and OSPF

- Configure Redistribution from OSPFv2 to EIGRP.

conf t

router eigrp 10

redistribute ospf 1 metric 10000 100 255 1 1500

- Configure Redistribution from EIGRP to OSPFv2.

router ospf 1

redistribute eigrp 10 subnets

- Verify OSPF, EIGRP, and redistribution settings.

show ip route eigrp

show ip protocols

show ip eigrp topology

show ip route ospf

show ip ospf

show ip ospf database

show run | section router

show ip protocols

Step 4: Configure IPv4 Static routing and Default Routes

- On MIU-GW and Branch_GW create IPv4 static default routes that point to ISP.

ip route 0.0.0.0 0.0.0.0 [Next-Hop-IP]

- On ISP, create IPv4 static routes that point to MIU-GW, Branch_GW, and Home Network.

ip route [Destination-Network] [Subnet-Mask] [Next-Hop-IP]

Part 5: Configure DHCP Server. (5%)

Step1: Configure DHCP Server on Main Branch

- Configure the excluded the first Three useable addresses from each address pool for VLANs: VLAN 5, VLAN 15, VLAN 130, and VLAN 35.

ip dhcp excluded-address 10.0.0.161 10.0.0.163

ip dhcp excluded-address 10.0.0.65 10.0.0.67

ip dhcp excluded-address 10.0.0.225 10.0.0.27

ip dhcp excluded-address 10.0.0.1 10.0.0.3

- Set the DHCP Server to assign IP address, DGW and DNS server for all Hosts in Main-Building and S-Building.

ip dhcp pool VLAN5

network 10.0.0.160 255.255.255.224

default-router 10.0.0.161

dns-server 10.0.1.21

ip dhcp pool VLAN15

network 10.0.0.64 255.255.255.192

default-router 10.0.0.65

dns-server 10.0.1.21

ip dhcp pool VLAN130

network 10.0.0.224 255.255.255.240

default-router 10.0.0.225

dns-server 10.0.1.21

ip dhcp pool VLAN35

network 10.0.0.0 255.255.255.192

default-router 10.0.0.1

dns-server 10.0.1.21

- Configure Main-MLS and S-MLS as a DHCP relay agent.

interface vlan 5

ip helper-address 10.0.1.18

interface vlan 15

ip helper-address 10.0.1.18

interface vlan 130

ip helper-address 10.0.1.18

interface vlan 35

ip helper-address 10.0.1.18

- Attempt to acquire an IP address from DHCP on PC1, PC2, PC3, PC4, PC5, and PC6.

On each PC:

- Set IP configuration to DHCP (Automatic).

- Use ipconfig (Windows) or equivalent to verify IP.

- Use ping to test gateway/DNS/internet.

Step2: Configure a Branch_GW Router as DHCP Server

  - Exclude the first five useable addresses from each address pool.

ip dhcp excluded-address 192.168.2.1 192.168.2.5

ip dhcp excluded-address 192.168.3.1 192.168.3.5

b.     Create a DHCP pool named B-LAN2 for VLAN2 and B-LAN3 for VLAN3. (case-sensitive).

§  Configure the DHCP pool to include the network address, the default gateway, and the IP address of the DNS server.

ip dhcp pool B-LAN2

network 192.168.2.0 255.255.255.0

default-router 192.168.2.1

dns-server 10.0.1.21

ip dhcp pool B-LAN3

network 192.168.3.0 255.255.255.0

default-router 192.168.3.1

dns-server 10.0.1.21

c.      Verify DHCP and Connectivity.

On PC11 (VLAN 2) and PC12 (VLAN 3):

- Set to DHCP.

- Run ipconfig or check IP settings.

- Test ping to:

  - Default gateway

  - Other PCs

  - Internet (if applicable)

Part 6: Configure Dynamic NAT with PAT and Static NAT. (5%)

Step 1: Configure Dynamic NAT with PAT on MIU-GW. The Public IP should use the third address from 209.165.200.224/28 address space.

Third Usable IP: 209.165.200.227

MIU-GW(config)# access-list 1 permit 10.0.0.0 0.255.255.255

MIU-GW(config)# ip nat pool publicPAT 209.165.200.227 209.165.200.227 netmask 255.255.255.240

MIU-GW(config)# ip nat inside source list 1 pool publicPAT overload

Step 2: Configure static NAT on MIU-GW to map the miu.edu.eg server inside address to the fourth address from the 209.165.200.224/28 address space.

Fourth Usable IP: 209.165.200.228

MIU-GW(config)# ip nat inside source static 10.0.1.20 209.165.200.228

Step 3: Configure the interfaces that will participate in NAT.

MIU-GW(config)# interface g0/0/0

MIU-GW(config-if)# ip nat inside

MIU-GW(config-if)# exit

MIU-GW(config)# interface g0/1/0

MIU-GW(config-if)# ip nat inside

MIU-GW(config-if)# exit

MIU-GW(config)# interface g0/2/0

MIU-GW(config-if)# ip nat inside

MIU-GW(config-if)# exit

MIU-GW(config)# interface g0/3/0

MIU-GW(config-if)# ip nat inside

MIU-GW(config-if)# exit

MIU-GW(config)# interface g0/1

MIU-GW(config-if)# ip nat inside

MIU-GW(config-if)# exit

MIU-GW(config)# interface g0/0

MIU-GW(config-if)# ip nat outside

MIU-GW(config-if)# exit

Step 4: Verify Dynamic NAT with PAT and static NAT Implementation.

MIU-GW# show ip nat translations

MIU-GW# show ip nat statistics

Step 5: Test Access with NAT and View NAT translations.

Part 7: Configure Network Management Features. (10%)

In this part, you will configure various network management features.

Step 1: Configure all devices. to use an NTP server.

configure terminal

ntp server 10.0.1.22

end

Router# show ntp status

Router# show association

Step 2: Configure and Verify Syslog on all devices.

configure terminal

logging host 10.0.1.22

logging trap debugging

logging on

end

show logging

Step 3: Web Server Configuration:

·         Configure the web server to display the website's main page with the MIU logo.

Configure the Web Server (10.0.1.20):

Click on the Web Server device

Go to the "Services" tab

Select "HTTP" service

Turn the service ON

In the "HTML Code" section, replace the default code with:

<html>

<head><title>MIU Website</title></head>

<body>

<h1>Welcome to MIU</h1>

<img src="/miu-logo.png" alt="MIU Logo">

</body>

</html>

Go to the "File" tab and upload an image named "miu-logo.png"

·         Ensure that the domain name for the website is miu.edu.eg.

- On the same server, go to the "DNS" service

- Add a new record:

  - Name: miu.edu.eg

  - Address: 10.0.1.20

  - Type: A Record

- Add another record:

  - Name: www

  - Address: 10.0.1.20

  - Type: CNAME Record

Step 4: Email Server Configuration:

·         Configure the email server properly to support sending and receiving emails via the domain miu.edu.eg.

- Configure the Email Server (10.0.1.19):

  - Click on the Email Server device

  - Go to the "Services" tab

  - Select "EMAIL" service

  - Configure domain: miu.edu.eg

  - Create user accounts (e.g., "admin", "user1") with passwords

  - Turn the service ON

- Configure DNS for Email:

  - On the DNS server (10.0.1.21), add these records:

    - Name: mail.miu.edu.eg

Step 5: DNS Server Configuration:

·         Configure the DNS server to resolve the domain name miu.edu.eg to the correct IP addresses for both the web server.

- Configure the DNS Server (10.0.1.21):

  - Click on the DNS Server device

  - Go to the "Services" tab

  - Select "DNS" service

  - Turn the service ON

  - Add these records:

    - Name: miu.edu.eg → 10.0.1.20 (A Record)

    - Name: www → miu.edu.eg (CNAME Record)

    - Name: mail → 10.0.1.19 (A Record)

      - Name: ns1 → 10.0.1.21 (A Record)

Part 8: Configure and Verify a Site-to-Site IPsec VPN. (10%)

Step 1: Verify connectivity throughout the network.

from MIU-GW to Branch_GW Public IP:

On MIU-GW: ping 64.100.1.0 (This is Branch_GW's public IP on the ISP side)

On MIU-GW: ping 209.165.200.226 (This is MIU-GW's public IP on the ISP side from Branch_GW's perspective).

Ping from MIU Internal Network to Branch Internal Network (e.g., from R-Building PC9 to PC11):

On a device in Branch VLAN2 (PC11): ping 10.0.0.164 PC1:

Verify Routing Tables:

show ip route on MIU-GW and Branch_GW to ensure they have routes to their respective internal networks and the ISP.

Step 2: Configure MIU-GW to support a site-to-site IPsec VPN with Branch_GW.

MIU-GW(config)# crypto isakmp policy 10

MIU-GW(config-isakmp)# encryption aes

MIU-GW(config-isakmp)# hash sha

MIU-GW(config-isakmp)# authentication pre-share

MIU-GW(config-isakmp)# group 2

MIU-GW(config-isakmp)# exit

MIU-GW(config)# crypto isakmp key CSC123 address 64.100.1.2

Branch_GW(config)# crypto isakmp policy 10

Branch_GW(config-isakmp)# authentication pre-share

Branch_GW(config-isakmp)# encryption aes

Branch_GW(config-isakmp)# hash sha

Branch_GW(config-isakmp)# group 2

Branch_GW(config-isakmp)# exit

Branch_GW(config)# crypto isakmp key myVPNkey address 209.165.200.226

MIU-GW(config)# crypto ipsec transform-set myTransformSet esp-aes esp-sha-hmac

MIU-GW(config)# ip access-list extended myTransformVPN

MIU-GW(config-ext-nacl)# permit ip 10.0.0.0 0.255.255.255 192.168.2.0 0.0.0.255

MIU-GW(config-ext-nacl)# permit ip 10.0.0.0 0.255.255.255 192.168.3.0 0.0.0.255

MIU-GW(config-ext-nacl)# exit

MIU-GW(config)# crypto map myMap 10 ipsec-isakmp

MIU-GW(config-crypto-map)# set peer 64.100.1.2

MIU-GW(config-crypto-map)# set transform-set myTransformSet

MIU-GW(config-crypto-map)# match address myTransformVPN

MIU-GW(config-crypto-map)# exit

Branch_GW(config)# crypto ipsec transform-set myTransformSet esp-aes esp-md5-hmac

Branch_GW(config)# ip access-list extended myTransformVPN

Branch_GW(config-ext-nacl)# permit ip 192.168.2.0 0.0.0.255 10.0.0.0 0.255.255.255

Branch_GW(config-ext-nacl)# permit ip 192.168.3.0 0.0.0.255 10.0.0.0 0.255.255.255

Branch_GW(config-ext-nacl)# exit

Branch_GW(config)# crypto map myMap 10 ipsec-isakmp

Branch_GW(config-crypto-map)# set peer 209.165.200.226

Branch_GW(config-crypto-map)# set transform-set myTransformSet

Branch_GW(config-crypto-map)# match address myTransformVPN

Branch_GW(config-crypto-map)# exit

Step 4: Verification Commands

show crypto isakmp sa

show crypto ipsec sa

show crypto engine connections active

Test connectivity through the VPN:

ping 192.168.50.1 source 10.0.1.1

Part 9: Configure a Wireless Home Router. (10%)

Step 1: Connect to a wireless router

Step 2: Configure the wireless router

### §  Configure the WR SSID.

### §  Change Network Name (SSID) to MIU-CSC230 for only 2.4 GHz. Notice that SSIDs are case-sensitive.

### §  Configure wireless security settings.

### §  Under the 2.4 GHz heading, select WPA2 Personal for the Security Mode.

### §  For the Encryption field, keep the default AES setting.

### §  In the Passphrase field, enter miu_csc230 as the passphrase.

Step 3: Connect the Wireless Clients and test the connectivity.

Part 10: Verify Connectivity. (10%)

Verify connectivity between all devices.

§  Perform a ping test between all devices connected to the network.

§  Take a screenshot of each ping operation.

§  Display the IP address of each interface (IP Interface) on each device.

§  Display the routing table for each router in the network.

§  Add all the information and screenshots to the documentation.

Part 11: Documentation and Handover. (10%)

- Prepare a final document, either in .docx or PPT format, with detailed configurations for each part of the implementation.

- If you face any issues, provide the troubleshooting guide that you follow to detect and resolve that issue in any step. (Optional)

- Include testing screenshots for the ping command and traceroute command on different PCs.

- Include the IP routing table for each device after testing is done and the routing is built dynamically.